TMS320F240 Serial Boot Loader

1.0 Introduction

This document describes the implementation of a serial boot loader for the TMS320F240 Digital Signal Processor.  The serial boot loader aims at in-situ programming of the flash array on the TMS320F240 device.

The TMS320F240 device has an on chip 16Kword flash array, for use as program memory. The Flash array may be programmed via a JTAG interface, however the JTAG interface needs extra interface circuitry and also the existence of the JTAG connector on the target. The serial boot loader is designed to program the flash memory array by making use of the Serial Communication Interface(SCI). The F240 SCI is an industry standard RS232 serial communication port and allows easy interface to a host PC communication port via an appropriate RS232 driver circuit. This serial boot loader provides a convenient technique to program the flash array through the serial interface which may be used for user functions during normal operation.

1.1	Overview

For this operation a minimal amount of boot loader code must be resident in the flash array. In this implementation it is proposed that the F24x flash array has resident only the comms kernel and sequencing algorithm. The actual Clear, Erase & Program routines are received from the Host during the programming sequence & executed by the sequencing s/w as required.  This allows the resident boot loader code to be small. Additionally, since the Clear, Erase, & Program algorithms are kept on the host, they can be updated in future version releases, without effecting the resident Flash s/w which is Programmed as part of Device Test.

Since the Comms + Sequencing s/w (Flash resident) is also Erased during the Programming sequence, it must be “re-loaded” as part of the User’s code. This is conveniently done by allowing the Host utility to send a copy of the Boot Loader code to the DSP at completion of the User’s code.

The F24x devices have a total of 544 Words of RAM, of which 256W can be used as Program memory.  The entire Programming sequence must therefore be completed by making use of only the available 256W of program memory.  This requires that the size of the comms kernel plus any of the Clear, Erase or Programming routines must execute entirely from 256W of program  memory.

A summary of the proposed Programming sequence is given below, given from the F24x DSP perspective:

- After boot-up, s/w control passes to the Comms kernel if the BIO pin is set high.

- If BIO is high, execute the Baud rate-detection routine to synchronize to Host comms baud rate.

- Comms kernel (excluding Baud rate-detection portion) “copies itself” to RAM B0.

- Receive CLEAR algorithm from Host & transfer it to RAM B0

- Execute CLEAR routine.

- Receive ERASE algorithm from Host & transfer it to RAM B0 over-writing previous CLEAR routine.

- Execute ERASE routine.

- Receive PROGRAM algorithm from Host & transfer it to RAM B0 over-writing previous ERASE routine.

- Execute PROGRAM routine. This is done by receiving data blocks from the Host, storing them temporarily in RAM B1 & then executing the PROGRAM routine.  This is repeated until all Data blocks have been downloaded from the Host.

- Receive Serial Boot loader code as another block & program it at address 3E00h.

Note: The Serial Boot loader code block is actually treated as just another User code block with it’s own destination address & length parameters, & hence it is transparent to the Comms Kernel & Sequencer running in B0.

2. Operation sequence - In detail

- At Boot-up Reset vector causes a branch to Comms kernel at location:
 3E00h (this allows up to 512 words for complete Boot loader including baud rate-detection routine & initialization. 

If BIO pin is high (+5v) initiate comms session with Host in preparation for Flash array Programming cycle. (i.e. proceed to #2)

If BIO pin is low (0v) pass control over to user’s code at 0040h.

- Auto-baud session with Host.
Comms kernel has no knowledge of F24x CLKIN frequency & hence the baud rate selection register bits must be determined by an Baud rate-detection algorithm, i.e.:

Host  side:
Host keeps sending same character (i.e. 0Dh, CR character) and waits until the correct acknowledge character (0AAh) from the Slave is sent.  Host waits for the acknowledge character for a defined time out period, after which the Host gives up & the comms session is aborted.

Slave side:
Slave sets baud rate control at “some” low initial setting and attempts to converge on the correct baud rate by receiving a character & comparing it to an expected character.  If different, the baud rate is increased to the next defined PLL setting.  Once a reliable “match” is made, an acknowledge character (0AAh) is sent to the Host to signify a baud rate lock has occurred.

Assumption:
CLKIN is in the range of 2 MHz  20 MHz & the Host baud rate is 38.4 kb/s

- Figure 2  shows a flow chart for  the Baud rate-detection process & the Table below shows the allowable CLKIN frequencies that can be used by the user.

| - Clkin frequency (MHz) | - PLL x | - DSP Clkout - (MHz) | - Actual BR - (Kbits/s) | - % ERROR |
| --- | --- | --- | --- | --- |
| - 20 | - 1 | - 20 | - 37.88 | - 1.36 |
| - 12 | - 1.5 | - 18 | - 38.79 | - -1.02 |
| - 10 | - 2 | - 20 | - 37.88 | - 1.36 |
| - 8 | - 2.5 | - 20 | - 37.88 | - 1.36 |
| - 6 | - 3 | - 18 | - 38.79 | - -1.02 |
| - 5 | - 4 | - 20 | - 37.88 | - 1.36 |
| - 4 | - 4.5 | - 18 | - 38.79 | - -1.02 |
| - 3.5795 | - 5 | - 17.8975 | - 38.57 | - -0.45 |
| - 2 | - 9 | - 18 | - 38.79 | - -1.02 |

- Self copy of Comms kernel to B0:
Main portion of Comms kernel plus the sequencer is copied from Flash array to B0.  The Auto-baud portion is no longer required and can be abandoned to save memory.  B0 is then selected as program memory at FF00h and control then passes to B0 in preparation for the CLEAR operation.

- Receive CLEAR Algorithm from Host.
Host sends:	- Start address (may not be necessary)
                           - Length (i.e. number of bytes to transfer)
                           - Actual algorithm code words, split into bytes.

Slave packs bytes into words & stores CLEAR routine in B0 just after the comms kernel itself.  Control is then passed to the CLEAR routine.

- Execute CLEAR routine:
If successful, send “OK” to Host, and proceed with ERASE.
If failure to Clear, send error code to Host & abort process.

- Receive ERASE algorithm from Host:
Host sends:	- Start address (may not be necessary)
                           - Length (i.e. number of bytes to transfer)
                           - Actual algorithm code words, split into bytes.

Slave packs bytes into words & stores ERASE routine in B0 just after the comms kernel itself, over-writing the previous CLEAR routine.  Control is then passed to the ERASE routine.

- Execute ERASE routine:
If successful, send “OK” to Host, and proceed with PROGRAM.
If failure to Erase, send error code to Host & abort process.

- Receive PROGRAM algorithm from Host:
Host sends:	- Start address (may not be necessary)
                           - Length (i.e. number of bytes to transfer)
                           - Actual algorithm code words, split into bytes.

Slave packs bytes into words & stores PROGRAM routine in B0 just after the comms kernel itself, over-writing the previous CLEAR routine.  Control is then passed to the PROGRAM routine.

- Execute PROGRAM sequence (1st cut version):
i)   Indicate to Host - “Ready to receive Data block”
ii)  Host sends Data block in following format:

Start Address

Size (number or bytes)

Last Block flag

Data block start o o o Data block end

Note:

- Max Data block size ~200 words.  Must be > 256 words.

- Start address is destination address in Flash array.

- If “Last block” flag is set then no further blocks are expected after this block.

- Data block is received in bytes then packed into words & stored on B1.

- Pass the Start address & Size (i.e. length of block) to the PROGRAM routine & execute.

- Send Pass / Fail status to Host.  If Fail, abort the process.

- If “Last block” flag is not set goto i),  else Programming sequence is complete.

Start

Is BIO=0 ?

Branch to User Code

Disable Watch Dog.

Initialize UART

Initialize Interrupts and

Memory Map.

Initialize the Baud Rate Parameter Table

Initialize the ‘Valid Baud Rate Counter’ (VBR)

A

Y

N

Figure 1. Flowchart for the Serial Boot Loader

Initialization.

A

Initialize PLL and Baud Rate Parameters

Call Set Clock Delay (~20ms)

Is the RX Flag Set ?

Fetch Character

N

Y

Is Char

=0x0D?

Increment Character Retry Count and set

Valid Baud Counter =0

Y

N

Increment VBR

Is CRC

>=max ?

N

Is VBC

>max ?

N

Y

Character Retry Count =0

and

inc Param_ptr

Y

B

Figure 2. Flowchart for the Serial Boot Loader  (contd.)

Baud Rate Detection Algorithm.

Fetch Header from Host

Get Clear algorithm from the host and

Execute clear algorithm.

Fetch Header

Get erase algorithm from the host and

Execute erase algorithm.

Fetch Header

Get program algorithm from the host

B

C

Figure 3 Flowchart for the Serial Boot Loader  (contd.).

C

Get Header from host

Get data block from host

Execute program algorithm

More data blocks to program ?

Y

N

N

End

Figure 4 Flowchart for the Serial Boot Loader  (contd.).